| Assessor | J Baxter / L Finch |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Date | 25.11.2025 |  |  |  |  |  |
| Assessment Type | Review |  |  |  |  |  |
| Suburb | Bayswater |  |  |  |  |  |
| Street/ Near Street | King William Street near Hill Street |  |  |  |  |  |
| COR File | 00373 94 F5 |  |  |  |  |  |
| School/s | Bayswater Primary School St Columba’s School |  |  |  |  |  |
| School Zone | No |  |  | Crossing Zone | No |  |
| Speed Limit | 40 Km/H (Permanent) |  |  | Method of Operation | Single |  |
| Number of Lanes | Vehicles |  | 2 | Other Lanes | Turning | No |
|  | Bus/Bike |  | 0 |  |  |  |
| Road Width | 10.8 M |  |  |  | Passing | No |
| Median/ Refuge/ Traffic Island | Yes Width 1.3 M |  |  | Cut out | Yes Width 2.4 M |  |
| Painted Lines | Yes | Median |  | Flashing Lights | On approach - Yes |  |
| Bends | N/A |  |  | Bends | Is line of sight affected? |  |
|  |  |  |  |  | N/A |  |
| Crest/ Incline/ Decline | Southeast Of Crossing |  |  | Crest/ Incline | Is line of sight affected? |  |
|  |  |  |  |  | No |  |
| Round About | Southeast Of Crossing |  |  | Round About | Is line of sight affected? |  |
|  |  |  |  |  | No |  |
| Symbolic sign distance | Northwest 115 M |  |  | Symbolic sign distance | Southeast 85.2 M |  |
| Tactiles used | Pram Ramps |  | No | Tactiles Condition | N/A |  |
|  | Centre of road |  | No |  |  |  |
| Sight Distance | Northwest 80-120 M |  |  | Sight Distance | Southeast 80-120 M |  |
| Bus Stops | Between 0-50 metres on approach of the crossing |  |  | Bus Stops | Between 0-50 metres on departure of the crossing |  |
|  | No |  |  |  | Yes (40m southeast) |  |
| Other Environmental Factors (i.e vegetation, power poles and parking restrictions) | - Parking restrictions on approaches (signs/road markings) - Yes - Trip Hazards - No - Distance from stop line to ped line school side 6.7 M - Distance from stop line to ped line non-school side 6.8 M - Stop lines through Cycle and Bus lanes - No - Painted lines condition - Worn - Ped lines - Yes |  |  |  |  |  |
| Hazards Observed | - Compliance of parking restrictions YES - Bus/Vehicle parking within 20 M on approach or 10 M on departure of crossing YES on both sides during crossing operation |  |  |  |  |  |
| Photo’s (10 as per arrows) |  |  |  |  |  |  |

* Note – site assessment must be completed while crossing is operational